Production of recombinant SARS COV-2 RBDSD1-Fc protein

I. Transient transection procedure for RBD-SD1 production

Materials, Reagents and Equipment

1.Expi293F™ Cells (Life Techn., Cat. #A14527)

2. Growth medium: Expi293™ Expression Medium 6 *1L (Life Techn. Cat. #   A1435102)

3. Opti-MEM I Reduced-Serum Medium (Gibco, Cat. #1850613)

4. FectoPRO® DNA transfection Kit 10mL (Polyplus-transfection® SA, Cat. #116-010)

5. Penicillin-streptomycin (Thermo Fisher Scientific, Gibco™, Cat# 15070063)

6. Sodium Butyrate (Sigma, Cat. # 303410)

7. Microbiological Safety Cabinet - Class II

8. CO2 shaking incubator

9.  Ultra Yield™ Flasks 2.5L.  Flask. (Thomson Instrument Company Cat.  #: 931136-B)

Procedure

Cell preparation:

- Four days prior to transfection calculate the total number of cells required for the scheduled volume of production. Subculture Expi293F™ cell to the final cell’s density 0.3-0.5mln/ml

- One day before transfection, split healthy dividing cells to the density of 1.3 × 106 viable cells/ml and maintain 18 – 24 hours prior to transfection to ensure that cells are at the log-phase growth at the time of transfection with a viability >95%.  Shake flasks overnight on an orbital shaker (95-125 rpm when using a shaker with a 2 cm orbital throw depending of the size of cell culture vessels) at 37°C, 8% CO2

- On the day of transfection, check cell density and viability using an automated cell counter or using the trypan blue dye exclusion method. Do not proceed with transfection unless cells are >95% viable.

Transient transfection:

- For each 1L of transfection, prepare reagent-DNA complexes as follows:

  - In a sterile tube, dilute 800ug of sterile plasmid DNA in 80ml of Opti-MEM® I Reduced Serum Medium (Life Technologies). Mix gently but thoroughly.

  - In a second sterile tube add 960ul of FectoPro transfection reagent

  - Add diluted DNA to the tube containing transfection reagent, invert several times and incubate at room temperature for 10 min to allow transfection complexes to form.

- Add 10 ml of sodium butyrate (from a 1 M stock solution) to 1L of the prepared cells

- After the DNA and FectoPro Transfection Reagent complex incubation is complete, add slowly transfection mix to each 1L of cells in the shaker flask.

- Shake flasks on an orbital shaker (95 rpm when using a shaker with a 2 cm orbital throw) at 30°C, 8% CO2 for 96 -120 hours of post transfection time.

- Start monitoring cells density and viability after 72 hours of post-transfection time. Media for secreted proteins may be harvested beginning at approximately 72 hours post-transfection and assayed for recombinant protein expression. The incubation time for optimal protein expression depends on the nature of the recombinant protein.

- Transfected cells for RBDSD1-Fc production were harvested for purification after 96 hours of post-transfection time

II. Purification protocol

Materials and Reagents

1. TALON® Metal Affinity Resin (Takara Bio USA, Inc. Cat#635652)

2. Slide-A-Lyzer™ Dialysis Cassette Kit, 10K MWCO, 12 mL (Thermo Fisher Scientific, Cat# 66807)

3. Amicon™ Ultra Centrifugal Filter Units, 15ml, 30kDa  (Millipore Sigma, Cat# UFC903024)

4. PreScission Protease, ( Product number: 27-0843-01,Cytiva)

5. Superdex™ 200 (prep grade) (GE Healthcare, Cat# 17-1043-01)

6. Gel Filtration Standards  (Bio-Rad, Cat#1511901)

Note: Day before of purification, prepare two sets of the clean centrifuge bottles, make sure no chemical residues left in it:

- Rinse already washed bottles with  dH20

- Fill again with  dH20 and leave for at least 3 hours. Discard water and let air-dry

Purification Buffers:

Note: All buffers should be passed through a 0.22 µm filter before use.

10x buffer: 0.2M Hepes 7.5, 3M NaCl , 1.5% Glycerol

1st Wash buffer: 20mM Hepes 7.5, 300mM NaCl, 5% glycerol, 0 mM Imidazole

2nd Wash buffer: 20mM Hepes 7.5, 300mM NaCl, 5% glycerol, 25mM Imidazole

Elution buffer: 20mM Hepes 7.5, 300mM NaCl, 5% glycerol, 300mM Imidazole

Dialysis Buffer: 20mM Hepes 7.5, 300mM NaCl, 5% glycerol

SEC Buffer: 20mM Hepes 7.5, 200mM NaCl, 5% glycerol (for some applications without glycerol)

Purification procedure, IMAC & Dialysis, Figure 1.

- Spin down cultured cells in centrifuge bottles @ 1500 RPM, 15 minutes at 40C

- Transfer cleared supernatant into centrifuge bottles and spin again @6500 RPM for 30min at 40C

- Transfer 1L of cleared supernatant into 2.8L Erlenmeyer flask, adjust pH by 10X times buffer, add Talon beads (2ml per L, 4ml slurry) and proceed to the batch absorption

- Batch absorption (BA) of cultured medium with Talon beads could be performed by using 2.8L Erlenmeyer flasks in Innova shaker at 40C for 2-3 hours, shaking at ~75 RPM

- Let bound resin to settle by tilting flask on the ice for 30 min. Carefully decant the supernatant with minimal disturbance to the beads into another flask, add fresh Talon beads and proceed with a second batch absorption.

- Collect all of the bound resin in a gravity flow column.

- Wash with 40-50CV buffer without imidazole

- Wash with 40CV buffer with 25mM imidazole, apply no more than 2-3 bed volumes at once (at this stage start checking on the Bradford assay, a protein might come out with even lower % of imidazole, collect washes)

- Elute with 300mM Imidazole:

- Don’t apply all EB at once; use not more than one bed volume each time and apply very slowly to avoid disturbance of the beads.

- Incubate for 3-5 min after a first elution buffer. Collect small fractions.

- The volume of elution buffer required to elute the protein may vary and hence the eluate should be constantly assayed by adding 10ul of the eluate to 100ul of Bradford reagent to determine the point at which no more protein is being eluted

Note: At this stage eluted proteins can be dialysed and freeze, or run on the SEC, depending on the applications and the nature of the purified proteins

- Combine peak fractions and dialyze, use at least 200x volume of the buffer to eliminate imidazole content.

- Use PRESCISSION PROTEASE if a cleavage of C terminal tag needed. Pass a protein over TALON® Metal Affinity Resin to remove cleaved tags and His-tagged protease

- Check concentration of dialyzed protein, concentrate if needed, make an aliquots and flash freeze before placing into -80OC

B. Purification procedure: SEC

1. Equilibrate Superdex 200 26/60 column with 20mM Hepes 7.5, 200mM NaCl, 5% glycerol buffer.

2. Combine the entire IMAC peak elution fraction, approximately 9 -12 mL of samples and Inject into the

pre -  equilibrated column.

4. Set flow rate to 1 mL/min.

5. Run gel with peak fractions on SDS-PAGE. Combine only the monomeric fractions.

6.  Concentrate protein in an Amicon™ Ultra Filter 15ml, 30kDa cut-off if necessary, make aliquots, flash

freeze, before placing into -800C